Research proposal · Teleop Research Platform · Draft, 29 September 2026

# Learned model-mediated teleoperation

A world model on a slow loop estimates what is around the arm. The existing 90 Hz predictors use that estimate to foresee contact and payload effects that the delayed operator cannot see yet.

Status Proposal only. Not an ADR and not a result. Nothing described here has been built or measured, and nothing here binds code. The first step of the plan is the ADR that would.

# What we propose

Today the operator sees the arm where a predictor extrapolates it from delayed robot poses. Near an obstacle or after a grasp, that prediction fails in a foreseeable way. It carries the arm through the surface and ignores the load, then corrects when the truth arrives. We propose a second, slow loop that watches the workspace through a fixed third-person camera and publishes a small description of the environment. The fast loop in Core applies that description to every frame's prediction.

| Slow loop robot site · RTX 4090 · about 1 Hz · output is seconds old |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Third-person camera fixed, calibrated to the robot base | → | Environment model Cosmos 3 Edge or a classical baseline | → | Environment state surfaces · held object · payload · confidence · frame time |  |

↓ Environment state crosses the impaired network link as a small message, through the same emulation as the pose stream ↓

| Fast loop operator side · Core · every 11.1 ms frame |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delayed robot pose 57-byte downlink | → | Command-aware predictor stronger baseline | → | Constraint step applies environment state | → | Reconciler existing axis | → | Quest display operator's headset |  |

exists today         to be built         environment state

Figure 1.  The two loops and what passes between them.

The slow loop owns only what changes more slowly than its output ages. A surface or a payload is still where it was two seconds ago. The arm's motion depends on what the operator does next, so it stays in the fast loop. The world model reports the environment, and the fast loop computes the correction.

# Where the platform stands

| What we have |  | What is missing |
| --- | --- | --- |
| - A deterministic, headless Core in C#, built by both .NET and Unity, with gates for replay determinism (verify) and architectural invariants (audit). - Operator-side predictors: passthrough, constant velocity, double exponential. - Five reconcilers, scored on correction magnitude, rate, jerk and time to convergence. - Playout buffering, and a composable impairment pipeline: fixed delay, jitter, Gilbert-Elliott burst loss, reordering, recorded delay traces. - A JetRover arm (four joints plus gripper) with cross-machine clock sync, and a Quest operator in Unity. - A 210-source literature review, an H200 for training and an RTX 4090 for inference. |  | - Environment sensing of any kind. The robot sends a 57-byte pose and nothing else. - The camera stream, its timestamps on the shared clock, and camera-to-base calibration. - A predictor that uses the commands already sent. IPredictor<Pose> observes only stamped robot poses. - An IInferenceBackend implementation, a plant that can make contact, and metrics windowed around contact and grasp events. - Physical motion-to-photon calibration (Gate 4). The comparisons proposed here are relative and do not depend on it. Absolute latency claims would. |

# Design choices and why

Table 1.  Design choices and the reason for each.

| Choice | Reason |
| --- | --- |
| The world model outputs environment state, never corrections. | A correction depends on the operator's next move, which a seconds-old model has not seen. Environment state survives the delay. |
| The fast loop stays analytic, inside Core. | The frame budget is 11.1 ms. Cosmos 3 Edge forward dynamics takes 3.91 s on an H100 (NVIDIA model card, 30 steps, undistilled). Core keeps its determinism and allocation rules. |
| The model runs at the robot site, next to the camera. | Video never crosses the delayed link. Only the small message does, through the same impairment emulation as the pose stream. |
| Model outputs are recorded as observations. | Replay stays bit-identical, and replaying a session needs no GPU. |
| A command-aware predictor comes first. | The operator side knows every command in flight. Without this baseline, some of the world model's measured gain would really be command information. |
| An oracle comes before Cosmos. | Ground-truth environment state, delivered at a swept update rate and age, bounds the benefit and sets the rate and age a real model must reach, before any training is paid for. |
| Constraints are confidence-gated, and false ones are counted. | A missed surface falls back to today's behaviour. A phantom surface stops the displayed arm while the real one keeps moving, which is worse. |
| Display only, at first. | Using constraints to limit commands belongs to the Autonomy axis, which needs closed-loop evaluation and is safety-relevant on hardware. |

# Plan

- ADR for the environment-state message, its Core contract, where the model runs, recording, and the new metrics. This is a new research question under the repository's rules, so the ADR comes before any code.

- Command-aware predictor, benchmarked against constant velocity and double exponential.

- Camera, timestamps and calibration. Record sessions with a soft obstacle and with grasps.

- Oracle sweep over update rate and age.  GO / NO-GO  Stop here if a perfect oracle at an achievable rate and age gives no benefit.

- Head-to-head in the slow slot: oracle ceiling, classical perception (depth, plane fitting, payload identification from joint data), and Cosmos 3 Edge fine-tuned on the H200 and served on the 4090.

The new metrics (error windowed around contact and grasp events, and a false-constraint rate) get their definitions in docs/metrics.md in the same pull request that first emits them.

# Risks

- The classical baseline may match Cosmos on surfaces and payloads. The learned model would then have to earn its place on pushed objects, deformables and occlusion.

- Calibration error lands directly in constraint positions, and the tolerances here are single-digit millimetres.

- Each world-model update moves the displayed arm. That is a model jump, and the reconcilers have never been tested against one.

- Contact data means driving the JetRover into things. Use a soft obstacle, a scripted just recipe, and a person watching.

# Prior work it builds on

Each source has a full entry, including the conditions it was measured under, in the project's docs/literature/ folder: the first five in predictive-display.md, the next five in world-models.md.

- Mitra & Niemeyer, “Model-mediated Telemanipulation,” IJRR 2008. The remote side abstracts sensing into a simple environment model that the operator side renders without lag. Shown on 1 DoF at a 4 s round trip.

- Park, “Improving Teleoperation with Models and Tasks,” Stanford PhD dissertation, 2009. MMT depends on the environment model changing slowly, and each update is where delay re-enters.

- Willaert, Van Brussel & Niemeyer, “Stability of Model-Mediated Teleoperation,” EuroHaptics 2012. Model jumps, introduced gradually (500 ms in, 300 ms out) behind a 4 mm deadband. 1 DoF, constant 150 ms.

- Bejczy, Kim & Venema, “The phantom robot,” ICRA 1990. A command-driven arm model overlaid on the camera view. Its reported benefit is in free motion.

- Black et al., “Visual-Haptic Model Mediated Teleoperation for Remote Ultrasound,” 2025. Adding a visual model to MMT compensated round trips up to 1000 ms for 15 operators (abstract only).

- Genie Envisioner (GE-Act), 2025. A 5 Hz world model drives a 30 Hz action stream.

- DSWAM, 2026. A fast executor is the default control path, a slow planner is activated when needed, and execution is asynchronous so the slow model never blocks control.

- AHEAD, 2026. A small latent world model with an uncertainty-gated horizon compensates a robot's own inference latency.

- Khalil & Kwon, 2026. No off-the-shelf video model meets real-time predictive display: 1.3–3.9 s per frame on an RTX 6000 Ada.

- MiraBench, 2026. Optimism bias is widespread across 12 world-model configurations.

- NVIDIA, Cosmos3-Edge model card. Forward dynamics takes 3.91 s on an H100 SXM 80GB at 480p, with 30 denoising steps, batch 1, undistilled. Accessed 29 September 2026; not yet in docs/literature/.

# Where it would extend prior work

- In the 210 sources reviewed, MMT's environment models are analytic, and no source uses a learned model of the scene for operator-facing delay compensation. World-model papers that mention teleoperation almost always mean collecting training data.

- MMT has been shown on 1 DoF, with haptics, at constant delay. This would be visual, in VR, on a multi-joint arm, under jitter, burst loss and reordering.

- The phantom robot helps in free motion. Contact is the case a command-driven model cannot foresee, and it is the case targeted here.

- MMT assumes the environment changes slowly, but no source reports how the benefit decays with update rate and age. The oracle sweep measures that curve.

- Willaert et al. state that longer lag enlarges model jumps without measuring it. This platform reports jump size and jerk as distributions.

- AHEAD-style compensation serves the robot's own compute, with no person in the loop. Here the delay is the network and the viewer is a person.

- A false-constraint rate would put a number on optimism bias as a display failure.

Figures from prior work were measured on other systems. They motivate these experiments and are not evidence about this platform. The Cosmos 3 Edge figure came from NVIDIA's model card through a summarising fetch and should be confirmed before reuse; the Cosmos 3 technical report was not read.